# Mathematical Model of the MSW Recovery-Pathway Screening

All equations used in the study, as implemented in the open-source package mswpath v3.2: data harmonisation, waste characterisation, life-cycle inventory and costs of six options, decision analysis, uncertainty, sensitivity, scenario discovery, value of information, break-even targets and the projection to 2045

Muhammad Fachri Ridwan — Supervisor: Prof. Anthony Halog — The University of Queensland — October 2026

How to read this document

- Equations are numbered in the order of the computation. Each section names the module and function that implements it, so every equation can be traced to code (mswpath/core.py unless stated otherwise).

- Everything is expressed per tonne of mixed MSW, wet weight, as received at the facility gate in 2025 (functional unit). Masses mj are tonnes of fraction j per tonne of MSW.

- In the Monte Carlo analysis every parameter is a vector of draws; all equations hold draw by draw.

- Numerical values are in the tables of Section 16, generated from data/*.csv, which also hold source and evidence status.

# 1 Notation and system

Indices: j ∈ {food, garden, paper, wood, textile, rubber, plastic, metal, glass, other} = the ten model fractions; k ∈ {SL, S1, …, S5} = options; i = Monte Carlo draw; p = carbon value (USD/t CO2e). Options: SL sanitary landfill with gas collection and flare; S1 waste-to-energy (grate incineration); S2 refuse-derived fuel (RDF) co-processed in the nearest cement kiln; S3 anaerobic digestion (AD) of food separated from mixed waste; S4 PHB bioplastic from landfill gas; S5 integrated RDF + AD. Baselines: open dump (OD) and SL. Every option manages the whole tonne; residues go to a residue landfill (sanitary unless stated). For each option the model returns Gk (kg CO2e/t), Ck (USD/t), fossil cumulative energy demand CEDk (MJ/t) and land take LUk (m2/t), with feasibility flags.

# 2 Data harmonisation to the 2025 baseline (MSWModel.harmonise, tonnage; build_database_v3.py)

## 2.1 Tonnage, projected once

Q2025=Qt (1+g) 2025−t,  M2025d=Mtd (1+g) 2025−t,  M2025n=Mtn (1+g) 2025−t  (1)

Qt is the reported tonnage (t/day) of year t, g the RIPS growth rate (or the default g0 of the register), d and n the domestic and non-domestic streams. The exponent is zero when the tonnage already refers to 2025; the model asserts Q used = Q2025 of the database (single-projection guard).

## 2.2 Composition

sj(ℓ)=xj(ℓ)j′​xj′(ℓ), f(ℓ)=100j′​xj′(ℓ),  sj=M2025d sj(d)+M2025n sj(n)M2025d+M2025n  (2)

x(ℓ)j is the wet-mass percentage of stream ℓ after mapping the RIPS categories to the model fractions (sum of the categories mapped to j); blank categories are absent (zero share), never imputed in the main case. Harmonisation rules (sampled priors): if organics are lumped, a share γ of food is moved to garden; if wood ≥ 10% with no leaf category, a share y of wood is moved to garden:

L: sgarden←sgarden+γ sfood, sfood←(1−γ) sfood;  W: sgarden←sgarden+y swood, swood←(1−y) swood  (3)

Scenario glass_imputed, only where glass is not reported, with an imputed share ug:

sj←sj (1−ug) (j≠glass),  sglass←sglass+ug  (4)

# 3 Waste characterisation (MSWModel.model)

Moisture as received wj, dry matter (1 − wj), dry lower heating value hj (legacy values multiplied by the calibration factor kh, plastic by kh,P), latent heat λ:

M=j​sjwj,  H=max0, j​sj (1−wj) kh,jhj−λM (MJ/kg)  (5)

Fossil CO2 released when a mass vector m is burned (Cj dry carbon, φj fossil share):

Efos(m)=103 4412j​mj (1−wj) Cj φj  (kg CO2/t)  (6)

Moisture enters once: DOC and C are on a dry basis and multiply dry mass (DOCwet = DOCdry(1−w)). In the Monte Carlo all fraction moistures move together through one common draw uw: wj = F−1tri(uw; wj,lo, wj, wj,hi).

# 4 Landfill module (open dump and sanitary landfill)

For a mass vector m sent to a landfill plus inert mass min (ash, digestate):

CH4(m)=103 kD MCF F 1612j​mj (1−wj) DOCj DOCfj  (kg CH4/t)  (7)

CH4capt=η CH4 [SL],  CH4emit=(CH4−CH4capt) (1−OX [SL])  (8)

GLF=CH4emit GWPCH4+aLF (t),  t=j​mj+min  (9)

CLF=t cu,  cu=cSLmax(Q t, 5)Qref,SL−eSL (SL),  cu=cOD (OD)  (10)

MCF = MCFSL (managed anaerobic) or MCFOD; [SL] = 1 for the sanitary landfill and 0 for the open dump (no collection, no cover oxidation); kD is a multiplier on DOC·DOCf; DOCf of rubber is the gap parameter DOCfR. The landfill baseline values are GSL = GLF(s), GOD = GLF(s) with OD settings; L0 = CH4(s) with MCF = 1 is reported for transparency.

# 5 Techno-economic building blocks

CRF=r (1+r)n(1+r)n−1,  ccap(Kref,qref,q)=CRF Kref (q/A/qref)b365 q (USD/t)  (11)

q = throughput (t/day) of the unit, A = availability (nameplate = throughput/A), b = capacity exponent, Kref the CAPEX at reference capacity qref. Electricity is credited at the grid factor EF = EFgrid kEF and valued at pel (or the Perpres 109/2025 tariff in that scenario where Q ≥ 1,000 t/day).

# 6 The six options

## 6.1 SL sanitary landfill with flare

GSL=GLF(s),  CSL=CLF(s)  (12)

## 6.2 S1 waste-to-energy

Eel=1033.6 H ηW (kWh/t),  G1=Efos(s)+nN2O GWPN2O+aW−Eel EF+GLF(0,α)  (13)

C1=ccap(KW,qW,Q)+oW−Eel pel+CLF(0,α)  (14)

α = bottom ash and APC residue (t/t) landfilled as inert mass.

## 6.3 S2 RDF to cement kiln

rj=sj min(τjkτ,1), min=j​rj, d=j​rj(1−wj), mout=minmin,d1−ω  (15)

e=j​rj(1−wj)kh,jhj−λ (mout−d)kNCV, enet=e−(min−mout) qdry, mdel=moutenete  (16)

NCVRDF=e/mout,  G2=Efos(r)+eR EF+aR+mdel D eftr−ψ enet EFcoal+GLF(s−r)  (17)

C2=ccap(KR,qR,Q)+oR+mdel D ctr−enet pRDF+CLF(s−r)  (18)

τj transfer coefficients to RDF, ω target moisture, qdry dryer heat per t of water, ψ GJ coal displaced per GJ RDF, D = road distance to the nearest kiln = great-circle distance × tortuosity, kNCV = 1 (0.78 in the stress test).

## 6.4 S3 anaerobic digestion of food separated from mixed waste

a=κ sfood, V=103 a (1−wfood) VSTS yCH4 kmech (Nm3/t), EAD=V(1−fAD)LHVCH43.6ηCHP(1−πAD)  (19)

G3=VρCH4fADGWPCH4+a aA−EADEF+GLF(s−a efood, δa)+π (eREF+aR)  (20)

C3=accap(KA,qA,Qa)+oA+cpre−EAD pel+CLF(s−a efood, δa)+πccap(KR,qR,Q)+oR  (21)

κ capture of food into the digester, kmech relative methane yield of mechanically separated food, cpre extra pre-treatment (USD/t feed), fAD fugitive methane share, πAD own electricity use, δ digestate landfilled per t feed, π front-end separation charged as a share of the RDF line; efood is the unit vector of the food fraction.

## 6.5 S4 PHB from landfill gas

PPHB=CH4capt(s)RPHB (kg/t), TPHB=PPHB Q 365103, cPHB(T)=c500max(T,1)500−ePHB  (22)

G4=GSL+PPHB (efPHB−σ efPP),  C4=CSL+PPHB (cPHB−pPHB)  (23)

## 6.6 S5 integrated RDF + AD

G5=GAD(a)+GRDF(s−a efood)+GLF(rej, δa),  C5=CAD(a)+CRDF(s−a efood)+CLF(rej, δa)  (24)

The whole tonne passes the RDF line, which also separates the food (no separate front-end charge); rej = (s − a efood) − r. Mass balance a + min + Σ rej = 1 is asserted.

# 7 Fossil cumulative energy demand and land take

PEF=3.6ηpp (1+ufuel) kEF (MJ/kWh),  MJd=1+udieselefdiesel (MJ per kg CO2e)  (25)

CED1=aW MJd−Eel PEF+CEDLF, CED2=eRPEF+aRMJd+mdelD eftrMJd−103ψenet(1+ucoal)+CEDLF  (26)

CEDAD=a aA MJd−EAD PEF,  CED4=CEDSL+PPHBefPHBefgas−σ CEDPP,  CEDLF=t aLF MJd  (27)

LUSL=j​mjρSLHSL+minρinHSLfgross, LUOD=j​mj+minρODHOD, LUplant=FPA 365 n  (28)

Biogenic energy is not counted. Plant land LUplant spreads the footprint FP over the lifetime throughput; each option’s land take is the sum of its plants and landfills.

# 8 Feasibility gates

G1: H ≥ 7 MJ/kg and G2: Q ≥ 150 t/day for WtE; G3: Q ≥ 1000 t/day for the Perpres 109/2025 tariff; G4: NCVRDF ≥ 12.56 MJ/kg and G5: D ≤ 300 km for RDF and RDF + AD; G6: TPHB ≥ 500 t/yr. An option failing a gate is infeasible in that draw.

# 9 Decision analysis (MSWModel.delta, decide, run_mc)

ΔGk=G0−Gk,  ΔCk=Ck−C0,  MACk=103 Ck−C0G0−Gk (0∈{OD,SL})  (29)

CICk(p)=Ck+p Gk103,  ki*(p)=argmink ∈ Fi CICik(p)  (30)

k Pareto efficient ⇔ no k′: Gk′≤Gk, Ck′≤Ck, (Gk′,Ck′)≠(Gk,Ck)  (31)

Pk(p)=1Ni=1N1ki*(p)=k,  SE=Pk(1−Pk)N  (32)

ℱi is the set of options feasible in draw i. CIC is the carbon-inclusive cost: it values greenhouse gases only. The carbon value is fixed at p = 0, 2, 25, 50 and 100 USD/t CO2e (policy choice; 2 = UU 7/2021 carbon tax). For two options with straight CIC lines the crossing carbon value is p* = 103(Cb−Ca)/(Ga−Gb).

# 10 Uncertainty propagation

x=Ftri−1(u;a,c,b)=a+u(b−a)(c−a)  if u<c−ab−a; b−(1−u)(b−a)(b−c)  otherwise, u∼U(0,1)  (33)

S∼Dir(α0 s+ϵ),  Sj=Γjj′​Γj′, Γj∼Gamma(α0sj+ϵ, 1)  (34)

Every register, CED/land and realism parameter is triangular (low, central, high). Composition is Dirichlet around the harmonised shares with heuristic concentration α0 (80, or 40 for flagged data); ε = 0 except in the pseudocount scenario (0.5). Each location uses its own random stream, seeded identically in every scenario (common random numbers), so differences between scenarios are paired. Scenarios replace parameter values (e.g. kmech = 1, cpre = 0) or the moisture vector (IPCC values; upper bounds) without changing the equations.

# 11 Sensitivity analysis

ρS(X,Y)=corrrank(X), rank(Y)  (mean |ρS| over locations)  (35)

S1,x=VarxE(Y|x)Var(Y),  ST,x=Ex∼Var(Y|x∼)Var(Y)  (36)

Sobol indices (discovery.sobol_indices) are estimated with Saltelli sampling and the SALib estimators (Saltelli et al. 2010 for S1, Jansen for ST), N = 512 base samples, composition fixed at each location. Outputs include the carbon-inclusive cost gap Y = CICS5(50) − CICSL(50).

# 12 Scenario discovery (mswpath/discovery.py)

α0=medianjsj(1−sj)Var(sj)−1  (method of moments over the 21 compositions)  (37)

Gini(node)=1−k​πk2,  split=argmaxGini(parent)−c​ncn Gini(c)  (38)

coverage=#{i∈B: ki*=k}#{i: ki*=k}, density=#{i∈B: ki*=k}#{i∈B}, mass=#{i∈B}N  (39)

40,000 condition sets are sampled (carbon value, tonnage, kiln distance, grid, tariff, source separation, composition, moisture, gas collection, landfill cost); the best option of each is explained by a CART tree (depth 4, Gini impurity) and PRIM boxes B, peeled by removing an α = 5% quantile of one condition (or fixing a binary condition) at a time to maximise density while coverage stays above a threshold.

# 13 Value of information (mswpath/voi.py)

NBik=−CICik(p),  EVPI=1Ni​maxkNBik−maxk1Ni​NBik  (40)

EVPPI(X)=1Ni​maxkgk(Xi)−maxk1Ni​NBik,  gk(X)≈ENBk | X  (41)

ĝk is a least-squares regression of NBk on X (cubic polynomial for one input; additive quadratic for a group of inputs resolved by one measurement campaign), after Strong et al. (2014). The EVPPI of a random input unrelated to the model is subtracted as a bias floor. Options feasible in fewer than half of the draws are removed. Value per year = EVPPI × 365 Q2025.

# 14 Break-even targets (mswpath/thresholds.py)

Δ(x)=CICk(p;x)−CICSL(p;x),  x*: Δ(x*)=0, x*=xa−Δ(xa)xb−xaΔ(xb)−Δ(xa)  (42)

One input x is swept over a grid with all others central; x* is found by linear interpolation between the grid points where Δ changes sign; ‘never’ / ‘always’ when Δ keeps one sign over the whole range.

# 15 Projection to 2045 (mswpath/projection.py)

EFgrid(t)=EFgrid(2025) max0, 1−δ (t−2025),  δ=135 yr−1 ⇒ EFgrid(2045)=0.429 EFgrid(2025)  (43)

x(t)=x(2025) (1+ex) t−2025: pel (e=0.02), oW,oR,oA (e=0.01), cSL,cOD (e=0.02)  (44)

Q(t)=Q(2025) (1+g) t−2025,  ΔYk=Yk(2045)−Yk(2025),  ΔYk=d​ΔYk(d)+Ik  (45)

All equations of Sections 3–9 are re-evaluated with these inputs; CAPEX is constant in real terms and composition, landfill gas and kiln coal are unchanged. ΔY(d) is the change with driver d (grid, costs, tonnage) applied alone and Ik the interaction. Local parameter values of the one-city tool are read as 2025 values and escalated in the same way.

# 16 Symbols and values

## 16.1 Constants (data/model_constants.csv)

| Key | Value | Unit | Meaning | Source |
| --- | --- | --- | --- | --- |
| GWP100_CH4 | 27 | - | GWP100 of non-fossil CH4 | IPCC AR6 WG1 Table 7.15 |
| GWP100_N2O | 273 | - | GWP100 of N2O | IPCC AR6 WG1 Table 7.15 |
| GWP20_CH4 | 79.7 | - | GWP20 of non-fossil CH4 | IPCC AR6 WG1 Table 7.15 |
| GWP20_N2O | 273 | - | N2O kept at GWP100 value in the GWP20 case | IPCC AR6 WG1 Table 7.15 |
| LAM | 2.443 | MJ/kg water | Latent heat of vaporisation | Physical constant |
| RHO_CH4 | 0.716 | kg/Nm3 | Density of methane | Physical constant |
| LHV_CH4 | 35.9 | MJ/Nm3 | Lower heating value of methane | Physical constant |
| AVAIL | 0.85 | - | Plant availability (nameplate = throughput / 0.85) | Analyst assumption |
| P_EL_PSEL | 0.2 | USD/kWh | Perpres 109/2025 WtE tariff | Perpres 109/2025 |
| PC_MAX | 100 | USD/t CO2e | Highest carbon value sampled | High-Level Commission on Carbon Prices 2017 |
| QREF_wte | 1000 | t/d | Reference capacity WtE | Azis et al. 2021 |
| QREF_rdf | 300 | t/d | Reference capacity RDF | GIZ ERiC-DKTI 2023 |
| QREF_ad | 100 | t/d | Reference capacity AD | GIZ 2017 in UNDP 2021 |
| QREF_sl | 500 | t/d | Reference capacity sanitary landfill | World Bank 2024 |
| GATE_lhv_min | 7 | MJ/kg | G1 WtE minimum LHV as received | Rand et al. 2000 |
| GATE_q_wte | 150 | t/d | G2 WtE minimum supply | Rand et al. 2000 |
| GATE_q_psel | 1000 | t/d | G3 PSEL tariff eligibility | Perpres 109/2025 |
| GATE_ncv_rdf | 12.56 | MJ/kg | G4 RDF minimum NCV (3,000 kcal/kg) | GIZ ERiC-DKTI 2023 |
| GATE_d_max | 300 | km | G5 maximum road distance to kiln | Analyst assumption |
| GATE_phb_min | 500 | t PHB/yr | G6 PHB minimum costed scale | Chidambarampadmavathy et al. 2017 |
| SEED | 42 | - | Random seed | Model setting |
| N_MC | 4000 | - | Monte Carlo draws per location and scenario | Model setting |
| BASE_YEAR | 2025 | - | Baseline year | Study design |

## 16.2 Grid emission factors (data/grid_emission_factors.csv)

| Grid | EF 2025 (kg CO2/kWh) | EF 2045 (projection) |
| --- | --- | --- |
| Jamali | 0.877 | 0.376 |
| Sumatera | 0.832 | 0.357 |
| Mahakam | 1.128 | 0.483 |

## 16.3 Fraction properties (data/table2_model_fraction_parameters.csv)

| Fraction | w as received (range) | w IPCC | DOC dry | DOCf | C dry | fossil φ | LHV dry MJ/kg | τ to RDF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| food | 0.75 (0.65-0.82) | 0.60 | 0.38 | 0.70 | 0.38 | 0.00 | 16.5 | 0.10 |
| garden | 0.55 (0.40-0.65) | 0.60 | 0.49 | 0.70 | 0.49 | 0.00 | 16.0 | 0.40 |
| paper | 0.35 (0.15-0.50) | 0.10 | 0.44 | 0.50 | 0.46 | 0.01 | 16.0 | 0.70 |
| wood | 0.25 (0.15-0.40) | 0.15 | 0.50 | 0.10 | 0.50 | 0.00 | 18.0 | 0.60 |
| textile | 0.35 (0.15-0.50) | 0.20 | 0.30 | 0.50 | 0.50 | 0.20 | 18.5 | 0.70 |
| rubber | 0.15 (0.05-0.25) | 0.16 | 0.47 | 0.00 | 0.67 | 0.20 | 23.0 | 0.60 |
| plastic | 0.22 (0.05-0.35) | 0.00 | 0.00 | 0.00 | 0.75 | 1.00 | 30.5 | 0.80 |
| metal | 0.05 (0.02-0.08) | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0 | 0.00 |
| glass | 0.02 (0.01-0.03) | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0 | 0.00 |
| other | 0.25 (0.10-0.40) | 0.10 | 0.00 | 0.00 | 0.03 | 1.00 | 0.0 | 0.10 |

## 16.4 Register parameters (data/assumption_register.csv)

| Key | Central (low–high) | Unit | Meaning | St. |
| --- | --- | --- | --- | --- |
| g_default | 0.0173 (0.0065–0.0254) | 1/yr | Tonnage growth rate where the RIPS gives none (median of six RIPS rates) | A |
| gamma | 0.27 (0.04–0.62) | - | Garden share of lumped ‘organik’ (median and range of 8 locations reporting both) | A |
| yard | 1 (0.5–1) | - | Share of ‘kayu’ treated as garden where kayu >= 10% and no leaf category exists | A |
| alpha_hi | 80 | - | Dirichlet concentration, unflagged composition (heuristic, not statistically calibrated) | A |
| alpha_lo | 40 | - | Dirichlet concentration, flagged composition (heuristic, not statistically calibrated) | A |
| F | 0.5 (0.475–0.525) | - | Methane fraction of landfill gas | L |
| doc_k | 1 (0.8–1.2) | x | Multiplier on DOC x DOCf (default uncertainty +-20%) | L |
| h_k | 0.9 (0.78–1.02) | x | Calibration factor on legacy dry-matter LHV (not applied to the Gotze plastic value) | L |
| h_plastic_k | 1 (0.9–1.1) | x | Uncertainty multiplier on plastic dry LHV (aggregate median, resin mix unknown) | A |
| efg_k | 1 (0.7–1.1) | x | Multiplier on grid emission factor (lower bound = decarbonising grid) | A |
| docf_rubber | 0 (0–0.5) | - | DOCf of rubber/leather: gap scenario (0 = not degradable; 0.5 = IPCC 2006 generic default) | A |
| mcf_od | 0.8 (0.4–0.8) | - | Methane correction factor, open dump | L |
| mcf_sl | 1 | - | Methane correction factor, managed anaerobic landfill | L |
| ox | 0.1 (0–0.1) | - | Oxidation in cover soil, sanitary landfill | L |
| cap | 0.5 (0.2–0.8) | - | Lifetime landfill-gas collection efficiency, sanitary landfill | A |
| anc_od | 1 (0–3) | kg CO2e/t | Diesel for dump operation | L |
| anc_sl | 5 (2–10) | kg CO2e/t | Diesel and materials for sanitary landfill operation | L |
| eta_wte | 0.18 (0.14–0.22) | - | Net electrical efficiency of grate incinerator on low-LHV waste | L |
| n2o_wte | 0.05 (0.02–0.08) | kg N2O/t | N2O from continuous stoker incineration | L |
| anc_wte | 10 (5–30) | kg CO2e/t | Auxiliary fuel and flue-gas reagents | L |
| ash | 0.25 (0.15–0.3) | t/t | Bottom ash and APC residue sent to landfill | L |
| tau_k | 1 (0.85–1.1) | x | Multiplier on RDF transfer coefficients | L |
| omega | 0.2 (0.15–0.25) | - | Target moisture of RDF (off-taker limit 20-25%) | L |
| q_dry | 3.2 (2.6–4) | GJ/t water | Heat to evaporate water in the dryer (self-supplied by burning RDF) | L |
| e_rdf | 40 (20–70) | kWh/t | Electricity for shredding, screening, baling | A |
| psi | 0.9 (0.8–1) | - | GJ coal displaced per GJ RDF in the kiln | L |
| ef_coal | 96.1 (94.6–101) | kg CO2/GJ | Combustion emission factor of displaced coal (sub-bituminous) | L |
| ef_truck | 0.1 (0.06–0.15) | kg CO2e/t-km | Truck transport of RDF incl. empty return | L |
| tort | 1.3 (1.2–1.5) | x | Road distance / straight-line distance to nearest kiln | A |
| anc_rdf | 3 (1–6) | kg CO2e/t | Diesel for loaders at the RDF plant | A |
| kappa | 0.7 (0.5–0.9) | - | Share of food waste captured into the digester | L |
| vs_ts | 0.87 (0.8–0.92) | - | Volatile solids / total solids of food waste | L |
| y_ch4 | 0.36 (0.28–0.44) | Nm3 CH4/kg VS | Methane yield of source-separated food waste realised in a digester (BMP-type parameter, not landfill DOCf) | V |
| eta_chp | 0.36 (0.32–0.4) | - | Electrical efficiency of biogas engine | L |
| par_ad | 0.2 (0.1–0.3) | - | Own electricity use (pulping, mixing, dewatering) | L |
| fug_ad | 0.05 (0.01–0.1) | - | Fugitive methane from the biogas plant (share of methane produced) | L |
| anc_ad | 3 (1–6) | kg CO2e/t | Diesel for loaders at the AD plant (per t feed) | A |
| pre | 0.4 (0.25–0.6) | - | Front-end separation of food from mixed waste: share of RDF-line cost and electricity | A |
| dig | 0.2 (0.1–0.35) | t/t feed | Dewatered digestate sent to the residue landfill | A |
| r_phb | 2.3 (2–2.8) | t CH4/t PHB | Methane per tonne PHB (1.79 as carbon + 0.52 for process energy) | L |
| ef_phb | 0.5 (0–1.5) | kg CO2e/kg | Nutrients and extraction chemicals | L |
| ef_pp | 1.9 (1.6–3.4) | kg CO2e/kg | Cradle-to-gate footprint of displaced polypropylene | L |
| sub_pp | 1 (0.7–1) | kg/kg | kg PP displaced per kg PHB | L |
| r | 0.1 (0.08–0.12) | 1/yr | Real discount rate, same for all pathways | A |
| n | 20 (15–30) | yr | Economic lifetime, same for all pathways | A |
| b | 0.7 (0.6–0.85) | - | Capacity exponent for CAPEX scaling, same for all units | L |
| K_wte | 120 M (100 M–200 M) | USD @1000 t/d | WtE CAPEX at reference capacity | V |
| o_wte | 27 (21–37) | USD/t | WtE O&M | V |
| K_rdf | 12.3 M (9 M–16 M) | USD @300 t/d | RDF plant CAPEX at reference capacity | L |
| o_rdf | 18.4 (12–24.5) | USD/t | RDF plant O&M | L |
| K_ad | 7 M (4 M–14 M) | USD @100 t/d | AD plant CAPEX at reference feed capacity | L |
| o_ad | 14 (10.6–30) | USD/t feed | AD plant O&M | L |
| c_sl | 20 (9–30) | USD/t @500 t/d | Full cost of sanitary landfill with gas collection and flare | L |
| e_sl | 0.3 (0.2–0.4) | - | Scale elasticity of landfill cost per tonne | L |
| c_od | 4 (2–8) | USD/t | Cost of operating an open dump | A |
| c_phb | 6.8 (4.8–9.6) | USD/kg @500 t/a | PHB production cost excl. methane feedstock | A |
| e_phb | 0.083 (0.05–0.15) | - | Scale elasticity of PHB unit cost | A |
| p_el | 0.07 (0.05–0.1) | USD/kWh | Market value of electricity (grid generation cost) | V |
| p_rdf | 1.15 (0.58–1.34) | USD/GJ | Price of RDF at kiln gate | L |
| p_phb | 4 (2.5–6) | USD/kg | Selling price of PHB | L |
| c_truck | 0.1 (0.05–0.15) | USD/t-km | Cost of trucking RDF | A |

## 16.5 Realism, scenario and stress parameters (data/scenario_parameters.csv)

| Key | Central (low–high) | Unit | Meaning | Use |
| --- | --- | --- | --- | --- |
| y_pen_mech | 0.6 (0.35–0.85) | - | Methane yield of food separated mechanically from mixed waste, relative to source-separated food | main case |
| pre_ofmsw | 5 (0–10) | USD/t feed | Extra pre-treatment of mechanically separated food (depackaging, removal of plastics and grit) | main case |
| glass_imp | 0 | - | Glass share imputed where glass is not reported (main case: none, i.e. treated as 0) | main case |
| glass_imp_dist | 0.012 (0.0069–0.0448) | - | Distribution used by the scenario glass_imputed (median, p10, p90 of the 15 locations that report glass separately) | scenario |
| pseudo | 0 | - | Dirichlet pseudocount added to every fraction (main case 0) | main case |
| pseudo_scen | 0.5 | - | Pseudocount used by the scenario dirichlet_pseudocount | scenario |
| rdf_ncv_k | 1 | x | Multiplier on the energy content of the delivered RDF (main case 1) | main case |
| rdf_ncv_k_scen | 0.78 | x | Stress test: RDF energy scaled so that its NCV falls to about 13-14 MJ/kg (lower than the 15-16.7 MJ/kg reported for Cilacap) | scenario |

## 16.6 CED and land-use factors (data/lcia_factors.csv)

| Key | Central (low–high) | Unit | Meaning | St. |
| --- | --- | --- | --- | --- |
| pef_eta | 0.32 (0.28–0.36) | - | Net thermal efficiency of the fossil generation displaced or consumed (primary energy = 3.6 / eta MJ per kWh) | L |
| up_fuel | 0.08 (0.03–0.15) | - | Upstream supply energy of power-plant fuel (mining, processing, transport) as a share of fuel energy | A |
| ef_diesel | 0.0741 | kg CO2/MJ | Converts diesel-type ancillary burdens (kg CO2e) to diesel energy; the ancillary terms also contain materials, so this is an approximation | L |
| up_diesel | 0.15 (0.1–0.25) | - | Upstream (refinery, crude supply) energy of diesel as a share of its energy | A |
| up_coal | 0.05 (0.02–0.1) | - | Upstream (mining, transport) energy of the kiln coal displaced by RDF | A |
| ef_natgas | 0.0561 | kg CO2/MJ | Converts the PHB nutrient/chemical burden (kg CO2e) to fossil MJ as if natural-gas based (proxy) | A |
| ced_pp | 75 (65–85) | MJ/kg | Cradle-to-gate fossil CED of displaced polypropylene incl. feedstock energy | L |
| rho_sl | 0.8 (0.6–1) | t/m3 | In-place density of compacted MSW in a sanitary landfill | A |
| h_sl | 20 (10–30) | m | Effective fill height of a sanitary landfill | A |
| f_gross_sl | 1.3 (1.1–1.5) | - | Gross site area / fill area (roads, leachate ponds, buffer) | A |
| rho_inert | 1.3 (1–1.6) | t/m3 | In-place density of ash, digestate cake and other inert residues | A |
| rho_od | 0.5 (0.35–0.7) | t/m3 | Density of uncompacted waste in an open dump | A |
| h_od | 5 (3–10) | m | Average heap height of an open dump | A |
| fp_wte | 40 (20–80) | m2 per t/d | Site footprint of a WtE plant per t/d nameplate capacity | A |
| fp_rdf | 50 (25–100) | m2 per t/d | Site footprint of an RDF plant (incl. drying and storage) per t/d | A |
| fp_ad | 60 (30–120) | m2 per t/d | Site footprint of an AD plant (incl. digestate handling) per t/d feed | A |

## 16.7 Projection parameters (data/projection_2045_parameters.csv)

| Key | Value | Unit | Meaning |
| --- | --- | --- | --- |
| base_year | 2025 | year | Year of the existing (baseline) condition |
| target_year | 2045 | year | Projection year |
| netzero_year | 2060 | year | Year in which the grid emission factor reaches zero |
| delta_grid | 0.028571428571428571 | 1/yr | Linear decline of the grid emission factor per year, delta = 1/35 (2025 to 2060): EF(t) = EF2025 x max(0, 1 - delta (t - 2025)); applied to GHG and fossil CED of grid electricity |
| esc_electricity | 0.02 | 1/yr | Real escalation of the market value of electricity (revenue of WtE and AD) |
| esc_labour | 0.01 | 1/yr | Real escalation of plant operation and maintenance cost (labour-driven O&M) |
| esc_landfill | 0.02 | 1/yr | Real escalation of landfill and open-dump cost (third rate of the 2/1/2% set; interpretation to be confirmed) |
| capex_escalation | 0 | 1/yr | CAPEX held constant in real terms |
| tonnage_growth | location | 1/yr | Tonnage grows from 2025 to 2045 at each location’s growth rate (RIPS rate or 1.73%/yr default); composition held constant (no defensible composition trend) |

## 16.8 Symbol map: equations to data keys

| Symbol | Data key |
| --- | --- |
| kD, F, MCFSL, MCFOD, η, OX | doc_k, F, mcf_sl, mcf_od, cap, ox |
| aLF, cSL, eSL, cOD | anc_sl / anc_od, c_sl, e_sl, c_od |
| kh, kh,P, DOCfR, kEF | h_k, h_plastic_k, docf_rubber, efg_k |
| ηW, nN2O, aW, α, KW, oW | eta_wte, n2o_wte, anc_wte, ash, K_wte, o_wte |
| kτ, ω, qdry, eR, ψ, EFcoal, eftr, aR, KR, oR, pRDF, ctr | tau_k, omega, q_dry, e_rdf, psi, ef_coal, ef_truck, anc_rdf, K_rdf, o_rdf, p_rdf, c_truck |
| κ, VS/TS, yCH4, kmech, cpre, fAD, ηCHP, πAD, δ, π, KA, oA | kappa, vs_ts, y_ch4, y_pen_mech, pre_ofmsw, fug_ad, eta_chp, par_ad, dig, pre, K_ad, o_ad |
| RPHB, c500, ePHB, pPHB, efPHB, efPP, σ | r_phb, c_phb, e_phb, p_phb, ef_phb, ef_pp, sub_pp |
| r, n, b, A, qref | r, n, b, AVAIL, QREF_* |
| ηpp, ufuel, udiesel, efdiesel, ucoal, CEDPP, ρ, H, fgross, FP | pef_eta, up_fuel, up_diesel, ef_diesel, up_coal, ced_pp, rho_*, h_*, f_gross_sl, fp_* |
| γ, y, g0, α0 | gamma, yard, g_default, alpha_hi / alpha_lo |

# References

IPCC (2006). Guidelines for National Greenhouse Gas Inventories, Vol. 5 Waste; IPCC (2019) Refinement; IPCC (2021) AR6 WG1 Table 7.15.

Saltelli, A., et al. (2010). Variance based sensitivity analysis of model output. Comput. Phys. Commun. 181, 259-270.

Breiman, L., Friedman, J.H., Olshen, R.A., Stone, C.J. (1984). Classification and Regression Trees. Wadsworth.

Friedman, J.H., Fisher, N.I. (1999). Bump hunting in high-dimensional data. Statistics and Computing 9, 123-143.

Strong, M., Oakley, J.E., Brennan, A. (2014). Estimating multiparameter partial expected value of perfect information from a probabilistic sensitivity analysis sample: a nonparametric regression approach. Med. Decis. Making 34, 311-326.

Frischknecht, R., et al. (2015). Cumulative energy demand in LCA: the energy harvested approach. Int. J. LCA 20, 957-969.

Full source list of every parameter: data/assumption_register.csv, data/secondary_data_verification.csv and MSW_Methodology_v3_EN.pdf.